Server Upgrade Options

Oaktree Law

Hardware Refresh Proposal

Replacement for Service Tag 2KPZZV2

April 15, 2026

Prepared For

Ed Pitts

Oaktree Law

Prepared By

Technijian, Inc.

18 Technology Dr., Ste 141  |  Irvine, CA 92618  |  949.379.8499

# 1. Executive Summary

Your current Dell PowerEdge server (service tag 2KPZZV2) is a 14th-generation platform that has been in service for several years and is approaching end-of-life for warranty, performance, and security patching.

In place of the previously discussed cloud migration, Technijian is proposing a hardware refresh to a newer-generation Dell PowerEdge server sourced through the Dell Outlet program (certified refurbished, full Dell warranty included).

This report presents three server candidates from Dell Outlet. All three:

- Equal or exceed the storage capacity of your current server

- Use modern Intel Xeon Scalable CPUs (dual socket vs. your current single socket)

- Include Dell's 3-Year Basic Hardware Warranty with Next Business Day onsite service

- Feature dual redundant power supplies matching your current setup

We have a clear recommendation (Option A — PowerEdge R760) but include two alternatives so you can weigh performance, capacity, and budget tradeoffs.

# 2. Your Current Server — Baseline Specs

The following specifications were decoded from the Dell bill of materials for service tag 2KPZZV2:

| Component | Current Configuration |
| --- | --- |
| Model | Dell PowerEdge (14th Generation — T440 / R440 class) |
| CPU | 1x Intel Xeon Scalable processor (single socket populated) |
| Memory | 32 GB DDR4-2666 RDIMM (2x 16 GB, dual rank) |
| Storage | 2x hard drives in RAID 1 |
| Data in use | ~900 GB (119 GB OS + 779 GB shared folders) |
| Power | Redundant 1+1 power supplies |
| Operating System | Windows Server 2019 Standard + CALs |
| Management | iDRAC9 Express |
| Optical | DVD±RW drive |

Why upgrade now:

- 14th-generation PowerEdge platforms are in the later portion of their support lifecycle

- DDR4-2666 memory is two generations behind current DDR5 speeds

- Single-socket configuration limits CPU performance for virtualization and future growth

- Warranty status should be verified — many 2KPZZV2-era servers are out of or near end-of-warranty

- Microsoft ends mainstream support for Windows Server 2019 in January 2029 — a logical moment to modernize both hardware and OS

# 3. Recommended Server Options

Option A (Recommended): Dell PowerEdge R760 — Balanced Upgrade

Dell Outlet SKU: POW0194278-R0031296-SA (Refurbished)

| Specification | This Server | vs. Current |
| --- | --- | --- |
| Generation | 16th Gen PowerEdge | +2 generations |
| CPU | 2x Intel Xeon Silver 4509Y (16 cores total, 4.10 GHz, 22.5 MB cache) | 2 sockets vs. 1, newer gen |
| RAM | 256 GB DDR5-5600 (4x 64 GB RDIMM, dual rank) | 8x RAM, DDR5 (faster) |
| Boot | 2x 480 GB PCIe M.2 NVMe (mirrored) | Dedicated boot separate from data |
| Data storage | 5x 1.2 TB 10K SAS + 960 GB SSD cache = ~4.8 TB usable RAID 5 | ~5x current usable storage |
| Networking | Broadcom 5720 quad-port 1 GbE + 2x Broadcom 57416 dual-port 10 GbE | Adds 10 GbE capability |
| Power | 800W (1+1) redundant | Same |
| Form factor | 2U rack, standard bezel | Standard data-center chassis |
| Warranty | 3-Year Basic Hardware Repair, 5x10 Next Business Day Onsite | Fresh warranty |

Why we like it:

- Right-sized upgrade without overspending. Five times the usable storage, eight times the RAM, and a modern DDR5 platform — but uses cost-effective Silver-tier CPUs rather than Gold/Platinum you don't need for a law firm file-server workload.

- True storage headroom. 4.8 TB usable gives ~5 years of growth room from your current 900 GB footprint.

- Newest generation available at Outlet pricing. 16th-generation Sapphire Rapids platform with full Dell warranty and certified refurbishment.

- Mirrored boot separation. Two dedicated M.2 NVMe drives for OS/boot keeps your data RAID independent — a modern best practice that your current server does not have.

Best fit if: You want meaningful performance and capacity gains, the newest generation available, and a warranty-backed refurbished deal.

Option B (Alternate): Dell PowerEdge R650 — Maximum CPU Power, Compact 1U

Dell Outlet SKU: POW0193711-R0031036-SA (Refurbished)

| Specification | This Server | vs. Current |
| --- | --- | --- |
| Generation | 15th Gen PowerEdge | +1 generation |
| CPU | 2x Intel Xeon Gold 6346 (32 cores total, 3.60 GHz, 36 MB cache, 205W) | Gold tier, 32 cores |
| RAM | 512 GB DDR4-3200 (16x 32 GB RDIMM, dual rank) | 16x RAM |
| Data storage | 2x 1.2 TB 10K SAS = 1.2 TB usable RAID 1 | ~20% more capacity |
| Networking | Broadcom 57414 10/25 GbE + 57504 Quad-port 10/25 GbE OCP | 25 GbE capability |
| Power | 800W (1+1) redundant | Same |
| Form factor | 1U rack, LCD bezel | Smaller footprint |
| Warranty | 3-Year Basic Hardware Repair, 5x10 NBD Onsite | Fresh warranty |

Why we like it:

- Heavy CPU and memory headroom. 32 cores and 512 GB RAM is future-proofing for virtualization, additional workloads (SQL, document management platforms), or any line-of-business software you may add.

- 1U footprint if rack space is a concern.

- 25 GbE networking — a significant jump if you're considering a modern switch upgrade.

Why we like it less than Option A:

- Minimal storage upgrade. 1.2 TB usable is only ~20% more than your current footprint — not much long-term headroom.

- 10K SAS spinning disks are slower than NVMe SSDs. For file-server workloads, disk speed often matters more than CPU.

- You pay for performance you won't use. 32 cores and 512 GB RAM is overkill for a small law firm file server.

Best fit if: You anticipate adding virtualized workloads, database servers, or document management software to the same box, and you don't need a large storage increase.

Option C (Alternate): Dell PowerEdge XR7620 — All-NVMe Premium

Dell Outlet SKU: POW0193712-R0028403-SA (Refurbished)

| Specification | This Server | vs. Current |
| --- | --- | --- |
| Generation | 16th Gen PowerEdge (rugged XR-series) | +2 generations |
| CPU | 2x Intel Xeon Gold 6426Y (32 cores total, 4.10 GHz, 37.5 MB cache) | Gold tier, 32 cores, high clock |
| RAM | 256 GB DDR5-5600 (16x 16 GB RDIMM, single rank) | 8x RAM, DDR5 |
| Data storage | 2x 3.84 TB PCIe U.2 NVMe (Gen 4) + 2x 480 GB M.2 boot = 3.84 TB usable RAID 1 | 4x capacity, all-flash NVMe |
| Networking | Broadcom 5720 quad-port 1 GbE OCP | Standard gigabit |
| Power | Redundant | Same |
| Form factor | 2U rugged chassis (NAF bezel, front-access) | Built for harsh environments |
| Warranty | 3-Year Basic Hardware Repair, 5x10 NBD Onsite | Fresh warranty |

Why we like it:

- All-flash NVMe storage. Fastest disk performance available — dramatic improvement over spinning SAS.

- Large, fast storage. 3.84 TB of NVMe handles your current workload with 4x headroom.

- Gold 6426Y CPUs run at higher clock (4.10 GHz) than the Silver parts in Option A — better single-threaded performance for applications.

Why we like it less than Option A:

- XR-series is a rugged/edge platform — built for deployment in industrial or tactical environments, not standard office racks. The chassis bezel and mounting are atypical for a law firm server room.

- Fewer networking ports than Option A (1 GbE only, no 10 GbE).

- Likely the most expensive of the three options due to premium CPU tier and all-NVMe storage.

Best fit if: Raw disk performance is critical (large case-file databases, document imaging, frequent large-file searches), you don't mind the rugged form factor, and budget is less of a constraint.

# 4. Side-by-Side Comparison

| Metric | Current 2KPZZV2 | Option A — R760 | Option B — R650 | Option C — XR7620 |
| --- | --- | --- | --- | --- |
| Generation | 14th Gen | 16th Gen | 15th Gen | 16th Gen |
| CPU sockets | 1 | 2 | 2 | 2 |
| Total cores | ~4-8 | 16 | 32 | 32 |
| CPU tier | Scalable | Silver | Gold | Gold |
| Memory | 32 GB DDR4 | 256 GB DDR5 | 512 GB DDR4 | 256 GB DDR5 |
| Usable storage | ~1-2 TB HDD | 4.8 TB SAS | 1.2 TB SAS | 3.84 TB NVMe |
| Storage type | HDD | SAS 10K | SAS 10K | NVMe all-flash |
| Boot separation | No | Yes (M.2) | No | Yes (M.2) |
| 10/25 GbE | No | 10 GbE | 25 GbE | 1 GbE only |
| Form factor | 1U/2U | 2U standard | 1U standard | 2U rugged |
| Warranty | Expired/near end | 3 yr fresh | 3 yr fresh | 3 yr fresh |

# 5. What Happens Next

1. Dell Outlet pricing request — Technijian sends Dell a formal quote request covering all three SKUs, including 10 Windows licenses, 5 RDP licenses, and both 3-year and 5-year ProSupport options.

2. You review pricing with our recommendation — we'll schedule a 30-minute call to walk through the numbers and confirm direction.

3. Order and staging (Week 1) — once approved, we place the order. Dell Outlet typically ships in 5-10 business days.

4. Migration project kickoff — Revised Statement of Work covering new server staging and OS install, data migration from old to new server, OneDrive/SharePoint migration for shared folders, Cloudbrink ZTNA for secure remote access, CrowdStrike / Huntress / Patch Management / MyRemote agent deployment, testing, and 30-day post-cutover support.

# 6. Our Recommendation

Option A — Dell PowerEdge R760 (SKU POW0194278-R0031296-SA) is the right fit for Oaktree Law.

It gives you:

- The newest generation available (16th Gen, DDR5)

- Five times your current storage capacity on properly-tiered SAS drives with a modern NVMe boot

- Eight times your current RAM

- 10 GbE networking for future-proofing

- A conventional rack form factor suited to your server room

- A fresh 3-year Dell warranty with next-business-day onsite repair

Options B and C are legitimate alternatives if your priorities differ (more CPU/RAM without storage growth for B; raw NVMe performance for C), but Option A best matches the workload profile of a law firm file server.

Questions on any of the options? Reply to this report or reach out directly and we'll walk through the details.